Set - I

======

- Design basic gates (AND,OR, NOT, XOR, XNOR, NAND, NOR) using VHDL.

X

Y

Z

0

0

0

0

1

0

1

0

0

1

1

1

X

Y

Z

0

0

0

0

1

1

1

0

1

1

1

1

X

Z

0

1

1

0

X

Y

Z

0

0

1

0

1

1

1

0

1

1

1

0

X

Y

Z

0

0

1

0

1

0

1

0

0

1

1

0

AND

OR

NOT

NAND

NOR

X

Y

Z

0

0

0

0

1

1

1

0

1

1

1

0

X

Y

Z

0

0

1

0

1

0

1

0

0

1

1

1

XOR

XNOR

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity GATES is

Port ( G1 : in std_logic;

G2 : in std_logic;

N1 : out std_logic;

A1 : out std_logic;

O1 : out std_logic;

X1 : out std_logic;

NA1 : out std_logic;

NO1 : out std_logic;

XN1 : out std_logic);

end GATES;

architecture Behavioral of GATES is

begin

N1 <= NOT G1;

A1 <= G1 AND G2;

O1 <= G1 OR G2;

X1 <= G1 XOR G2;

NA1 <= G1 NAND G2;

NO1 <= G1 NOR G2;

XN1 <= G1 XNOR G2;

end Behavioral;

- (a) Design a Half Adder using dataflow modeling with VHDL.

S= X Y

C= X.Y

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity HALF_ADDER is

Port ( HA1 : in std_logic;

HA2 : in std_logic;

HA_SUM : out std_logic;

HA_CAR : out std_logic);

end HALF_ADDER;

architecture Behavioral of HALF_ADDER is

begin

HA_SUM <= HA1 XOR HA2;

HA_CAR <= HA1 AND HA2;

end Behavioral;

2. (b) Design a Half Adder using structural modeling

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity HALF_ADDER_COMP is

Port(HA1 : in std_logic;

HA2 : in std_logic;

HA_SUM : out std_logic;

HA_CRY : out std_logic);

end HALF_ADDER_COMP;

architecture Behavioral of HALF_ADDER_COMP is

component XOR_TWO is

Port ( 	X : in std_logic;

Y : in std_logic;

Z : out std_logic);

end component XOR_TWO;

component AND_TWO is

Port( 	A : in std_logic;

B : in std_logic;

C : out std_logic);

end component AND_TWO;

begin

LB1 : XOR_TWO PORT MAP(HA1,HA2,HA_SUM);

LB2 : AND_TWO PORT MAP(HA1,HA2,HA_CRY);

end Behavioral;

3. (a) Design a Full adder using dataflow modeling.

D= X Y  Z

C= X.Y+Z(X Y)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity FULL_ADDER_DATA is

Port ( FA1 : in std_logic;

FA2 : in std_logic;

FA3 : in std_logic;

FA_SUM : out std_logic;

FA_CAR : out std_logic);

end FULL_ADDER_DATA;

architecture Behavioral of FULL_ADDER_DATA is

begin

FA_SUM <= FA1 XOR FA2 XOR FA3;

FA_CAR <= (FA1 AND FA2) OR (FA2 AND FA3) OR(FA1 AND FA3);

end Behavioral;

3. (b) Design a Full adder using the macro of Half adder.

HA

HA

X

Y

Z

C

S

D= X Y  Z

C= X.Y+Z(X Y)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity FULL_ADDER_COMP is

Port ( 	FA1 : in std_logic;

FA2 : in std_logic;

FA3 : in std_logic;

FA_SUM : out std_logic;

FA_CAR : out std_logic);

end FULL_ADDER_COMP;

architecture Behavioral of FULL_ADDER_COMP is

component HALF_ADDER is

Port ( 	HA1 : in std_logic;

HA2 : in std_logic;

HA_SUM : out std_logic;

HA_CAR : out std_logic);

end component HALF_ADDER;

signal T1,T2,T3,SUM : std_logic;

begin

F1: HALF_ADDER port map(FA1,FA2,T1,T2);

F2:	HALF_ADDER port map(T1,FA3,SUM,T3);

FA_CAR <= (T2 OR T3);

FA_SUM <= SUM;

end Behavioral;

4. (a) Design a 4-bit adder circuit using the macro of Full adder.

Cout  C3   C2   C1   Cin

A3   A2   A1  A0

B3    B2   B1  B0

S3     S2   S1   S0

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity FULL_ADDER_4BIT is

Port(Ain : in std_logic_vector(3 downto 0);

Bin : in std_logic_vector(3 downto 0);

Cary_in : in std_logic;

Cout : out std_logic_vector(3 downto 0);

Cary_out : out std_logic);

end FULL_ADDER_4BIT;

architecture Behavioral of FULL_ADDER_4BIT is

component FULL_ADDER_COMP is

Port(	FA1 : in std_logic;

FA2 : in std_logic;

FA3 : in std_logic;

FA_SUM : out std_logic;

FA_CAR : out std_logic);

end component FULL_ADDER_COMP;

signal T : std_logic_vector(4 downto 0);

begin

T(0) <= Cary_in;

FA4B_1: FULL_ADDER_COMP PORT MAP(Ain(0),Bin(0),T(0),Cout(0),T(1));

FA4B_2: FULL_ADDER_COMP PORT MAP(Ain(1),Bin(1),T(1),Cout(1),T(2));

FA4B_3: FULL_ADDER_COMP PORT MAP(Ain(2),Bin(2),T(2),Cout(2),T(3));

FA4B_4: FULL_ADDER_COMP PORT MAP(Ain(3),Bin(3),T(3),Cout(3),T(4));

Cary_out <= T(4);

end Behavioral;

4. (b) Design a 4-bit adder circuit using carry look-ahead adder circuit.

carry propagate: Pi = Ai XOR Bi

carry generate: Gi = AiBi

sum: Si = Pi XOR Ci

carry: Ci+1 = Gi+PiCi

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity CARRY_LOOK_AHEAD_ADDER_4BIT is

port(	CAin1 : in std_logic_vector(3 downto 0);

CAin2 : in std_logic_vector(3 downto 0);

CAcarryin : in std_logic;

CAsum : out std_logic_vector(3 downto 0);

CAcarry : out std_logic);

End entity CARRY_LOOK_AHEAD_ADDER_4BIT;

Architecture CLA_ADDER OF CARRY_LOOK_AHEAD_ADDER_4BIT is

Begin

process(CAin1,CAin2,CAcarryin)

variable P,G : std_logic_vector(4 downto 1);

variable C : std_logic_vector(4 downto 0);

begin

C(0) := CAcarryin;

for I in 1 to 4 loop

G(I) := (CAin1(I-1) AND CAin2(I-1));

P(I) := (CAin1(I-1) XOR CAin2(I-1));

C(I) := G(I) OR (P(I) AND C(I-1));

End loop;

for I in 0 to 3 loop

CAsum(I) <= P(I+1) xor C(I);

End loop;

CAcarry <= C(4);

end process;

End CLA_ADDER;

5. (a) Design a Half Subtractor using data flow modeling.

D= X Y

C= A.B

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity half_subtractor is

Port(HS1 : in std_logic;

HS2 : in std_logic;

HS_SUB : out std_logic;

HS_BOR : out std_logic);

end half_subtractor;

architecture Behavioral of half_subtractor is

begin

HS_SUB <= HS1 XOR HS2;

HS_BOR <= (NOT HS1) AND HS2;

end Behavioral;

5. (b) Deisgn a full subtractor using a 3:8 Decoder circuit and OR gates.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity FULL_SUBTRACTOR_38DECODER is

Port(FS1 : in std_logic;

FS2 : in std_logic;

FS3 : in std_logic;

FS_SUB : out std_logic;

FS_BOR : out std_logic);

end FULL_SUBTRACTOR_38DECODER;

architecture Behavioral of FULL_SUBTRACTOR_38DECODER is

component DECODER_38 is

Port(	S  : in std_logic_vector(2 downto 0);

D  : out std_logic_vector(7 downto 0);

en : in std_logic );

end component DECODER_38;

signal T : std_logic_vector(7 downto 0);

signal Temp : std_logic_vector(2 downto 0);

begin

Temp <= FS1 & FS2 & FS3;

DE1 : DECODER_38 PORT MAP(Temp,T,'1');

FS_SUB <= T(1) or T(2) or T(4) or T(7);

FS_BOR <=	T(1) or T(2) or T(3) or T(7);

end Behavioral;

6. Design a BCD adder circuit using VHDL.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity BCD_ADDER is

Port(	BCD1 : in std_logic_vector(3 downto 0);

BCD2 : in std_logic_vector(3 downto 0);

BCD_CIN : in std_logic;

BCD_SUM : out std_logic_vector(3 downto 0);

BCD_COUT : out std_logic);

end BCD_ADDER;

architecture Behavioral of BCD_ADDER is

component FULL_ADDER_COMP is

Port(	FA1 : in std_logic;

FA2 : in std_logic;

FA3 : in std_logic;

FA_SUM : out std_logic;

FA_CAR : out std_logic);

end component FULL_ADDER_COMP;

signal TS : std_logic_vector(3 downto 1);

signal C : std_logic_vector(6 downto 0);

signal T : std_logic_vector(2 downto 0);

signal Cin : std_logic;

begin

C(0) <= BCD_CIN;

Cin <= '0';

B1: FULL_ADDER_COMP port map(BCD1(0), BCD2(0), C(0), BCD_SUM(0), C(1));

B2: FULL_ADDER_COMP port map(BCD1(1), BCD2(1),C(1),TS(1),C(2));

B3: FULL_ADDER_COMP port map(BCD1(2), BCD2(2),C(2),TS(2),C(3));

B4: FULL_ADDER_COMP port map(BCD1(3), BCD2(3),C(3),TS(3),C(4));

T(0) <= TS(3) and TS(2);

T(1) <= TS(3) and TS(1);

T(2) <= C(4) or T(0) or T(1);

B5: FULL_ADDER_COMP port map(TS(1),T(2),Cin,BCD_SUM(1),C(5));

B6: FULL_ADDER_COMP port map(TS(2),T(2),C(5),BCD_SUM(2),C(6));

BCD_SUM(3) <= TS(3) xor C(6);

BCD_COUT <= T(2);

end Behavioral;

Set - II

=====

1. Design a 4-bit adder using generate statement.

Cout  C3   C2   C1   Cin

A3   A2   A1  A0

B3    B2   B1  B0

S3     S2   S1   S0

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity FULLADDER_4BIT_GENERATE is

Port ( 	FAIN_A : in std_logic_vector(3 downto 0);

FAIN_B : in std_logic_vector(3 downto 0);

FAIN_C : in std_logic;

FA_SM : out std_logic_vector(3 downto 0);

FA_CY : out std_logic);

end FULLADDER_4BIT_GENERATE;

architecture Behavioral of FULLADDER_4BIT_GENERATE is

component FULL_ADDER_COMP is

Port ( 	FA1 : in std_logic;

FA2 : in std_logic;

FA3 : in std_logic;

FA_SUM : out std_logic;

FA_CAR : out std_logic);

end component FULL_ADDER_COMP;

signal C : std_logic_vector(4 downto 0);

begin

C(0) <= FAIN_C;

GK: for k in 3 downto 0 generate

U1: FULL_ADDER_COMP PORT MAP (FAIN_A(K), FAIN_B(K), C(K), FA_SM(K), C(K+1));

end generate gk;

FA_CY <= C(4);

end Behavioral;

2. Design a mode-control 4-bit adder/subtractor circuit.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity FULL_ADDER_4_MODE is

Port ( 	FAIN_A : in std_logic_vector(3 downto 0);

FAIN_B : in std_logic_vector(3 downto 0);

FA_MODE : in std_logic;

FA_SUM : out std_logic_vector(3 downto 0);

FA_CAR : out std_logic;

FA_V : out std_logic);

end FULL_ADDER_4_MODE;

architecture Behavioral of FULL_ADDER_4_MODE is

component FULL_ADDER_COMP is

Port ( 	FA1 : in std_logic;

FA2 : in std_logic;

FA3 : in std_logic;

FA_SUM : out std_logic;

FA_CAR : out std_logic);

end component FULL_ADDER_COMP;

signal carry : std_logic_vector(4 downto 0);

signal ModeB : std_logic_vector(3 downto 0);

begin

ModeB(0) <=  FAIN_B(0) xor FA_MODE;

ModeB(1) <=  FAIN_B(1) xor FA_MODE;

ModeB(2) <=  FAIN_B(2) xor FA_MODE;

ModeB(3) <=  FAIN_B(3) xor FA_MODE;

carry(0) <= FA_MODE;

FA4M_1: FULL_ADDER_COMP PORT MAP(FAIN_A(0), ModeB(0), carry(0), FA_SUM(0), carry(1));

FA4M_2: FULL_ADDER_COMP PORT MAP(FAIN_A(1), ModeB(1), carry(1), FA_SUM(1), carry(2));

FA4M_3: FULL_ADDER_COMP PORT MAP(FAIN_A(2), ModeB(2), carry(2), FA_SUM(2), carry(3));

FA4M_4: FULL_ADDER_COMP PORT MAP(FAIN_A(3), ModeB(3), carry(3), FA_SUM(3),carry(4));

FA_V <= carry(4) xor carry(3);

FA_CAR <= carry(4);

end Behavioral;

3. (a) Design a 4:1 Multiplexer using if_else statement.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity MUX4_1_IF_ELSE is

Port(	MUXIN_0,MUXIN_1,MUXIN_2,MUXIN_3 : in std_logic;

MUXSEL : in std_logic_vector(1 downto 0);

MUXOUT : out std_logic);

end MUX4_1_IF_ELSE;

architecture Behavioral of MUX4_1_IF_ELSE is

begin

PROCESS(MUXSEL)

BEGIN

IF MUXSEL = "00" THEN

MUXOUT <= MUXIN_0;

ELSIF MUXSEL = "01" THEN

MUXOUT <= MUXIN_1;

ELSIF MUXSEL = "10" THEN

MUXOUT <= MUXIN_2;

ELSE

MUXOUT <= MUXIN_3;

END IF;

END PROCESS;

end Behavioral;

3. (b) Design a 4:1 Multiplexer using case statement.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity MUX4_1_CASE is

Port(	MUXIN_0,MUXIN_1,MUXIN_2,MUXIN_3 : in std_logic;

MUXSEL : in std_logic_vector(1 downto 0);

MUXOUT : out std_logic);

end MUX4_1_CASE;

architecture Behavioral of MUX4_1_CASE is

begin

PROCESS(MUXSEL)

BEGIN

CASE MUXSEL IS

WHEN "00" => MUXOUT <= MUXIN_0;

WHEN "01" => MUXOUT <= MUXIN_1;

WHEN "10" => MUXOUT <= MUXIN_2;

WHEN OTHERS => MUXOUT <= MUXIN_3;

END CASE;

END PROCESS;

end Behavioral;

4. (a) Design a 2:4 Decoder using behavioral modeling.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity DECODER2_4 is

Port ( 	SE : in std_logic_vector(1 downto 0);

DE : out std_logic_vector(3 downto 0));

end DECODER2_4;

architecture Behavioral of DECODER2_4 is

signal TE : std_logic_vector(3 downto 0);

begin

TE <= 	"0001" when SE = "00" else

"0010" when SE = "01" else

"0100" when SE = "10" else

"1000" when SE = "11";

DE <= TE;

end Behavioral;

4. (b) Design a 3:8 Decoder using behavioral modeling.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity DECODER_38 is

Port(S : in std_logic_vector(2 downto 0);

D : out std_logic_vector(7 downto 0);

en : in std_logic );

end DECODER_38;

architecture Behavioral of DECODER_38 is

begin

------------------process 1---------------

process(en,S)

begin

if en ='1' then

case s is

when "000" =>D<="00000001";

when "001" =>D<="00000010";

when "010" =>D<="00000100";

when "011" =>D<="00001000";

when "100" =>D<="00010000";

when "101" =>D<="00100000";

when "110" =>D<="01000000";

when "111" =>D<="10000000";

when others => null;

end case;

else

d<=(others =>'0');

end if;

end process;

---------------------------------------

end Behavioral;

4. (c) Design a 5:32 decoder using the macro of above two design.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity DECODER5_32_COMP2_4_3_8 is

port(	SEL : in std_logic_vector(4 downto 0);

DEC : out std_logic_vector(31 downto 0));

end DECODER5_32_COMP2_4_3_8;

architecture Behavioral of DECODER5_32_COMP2_4_3_8 is

component DECODER2_4 is

Port(	SE : in std_logic_vector(1 downto 0);

DE : out std_logic_vector(3 downto 0));

end component DECODER2_4;

component DECODER_38 is

Port(	S : in std_logic_vector(2 downto 0);

D : out std_logic_vector(7 downto 0);

en : in std_logic );

end component DECODER_38;

signal EN_DE : std_logic_vector(3 downto 0);

signal T : std_logic_vector(31 downto 0);

begin

D1 : DECODER2_4 PORT MAP(SEL(4 downto 3),EN_DE);

D2 :	DECODER_38 PORT MAP(SEL(2 downto 0),T(7 downto 0), EN_DE(0));

D3 :	DECODER_38 PORT MAP(SEL(2 downto 0),T(15 downto 8), EN_DE(1));

D4 :	DECODER_38 PORT MAP(SEL(2 downto 0),T(23 downto 16), EN_DE(2));

D5 :	DECODER_38 PORT MAP(SEL(2 downto 0),T(31 downto 24), EN_DE(3));

DEC <= T;

end Behavioral;

5. Design 4:2 binary encoder using VHDL.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity ENCODER4_2 is

Port( ENin  : in std_logic_vector(3 downto 0);

ENout : out std_logic_vector(1 downto 0));

end ENCODER4_2;

architecture Behavioral of ENCODER4_2 is

begin

process(ENin)

begin

case ENin is

when "0001" => ENout <= "00";

when "0010" => ENout <= "01";

when "0100" => ENout <= "10";

when "1000" => ENout <= "11";

when others => NULL;

end case;

end process;

end Behavioral;

6. Design a Even-parity generator and cheker circuit.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity PARITY_CHECKER_GENERATOR_8BIT is

Port(	Pin : in std_logic_vector(7 downto 0);

Pcheck : out std_logic;

Pgenerate : out std_logic);

end PARITY_CHECKER_GENERATOR_8BIT;

architecture Behavioral of PARITY_CHECKER_GENERATOR_8BIT is

signal T : std_logic_vector(7 downto 0);

begin

T(0) <= Pin(0) XOR Pin(1);

T(1) <= Pin(2) XOR Pin(3);

T(2) <= Pin(4) XOR Pin(5);

T(3) <= Pin(6) XOR Pin(7);

T(4)	<= T(0) XOR T(1);

T(5)	<= T(2) XOR T(3);

T(6)	<= T(4) XOR T(5);

T(7)	<= NOT T(6);

Pcheck <= T(7);

Pgenerate <= T(6);

end Behavioral;

7. Design a 4-bit by 3-bit binary multiplier circuit using the macro of 4-bit adder and AND gates.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Mutiplier4_3 is

port(	B : in std_logic_vector(3 downto 0);

A : in std_logic_vector(2 downto 0);

O : out std_logic_vector(6 downto 0));

end Mutiplier4_3;

architecture Behavioral of Mutiplier4_3 is

component FULL_ADDER_4BIT is

Port(	Ain : in std_logic_vector(3 downto 0);

Bin : in std_logic_vector(3 downto 0);

Cary_in : in std_logic;

Cout : out std_logic_vector(3 downto 0);

Cary_out : out std_logic);

end component FULL_ADDER_4BIT;

signal C : std_logic;

signal T : std_logic_vector(11 downto 0);

signal A1,B1,C1,D1,E1 : std_logic_vector(3 downto 0);

begin

T(0) <= A(0) and B(0);

T(1) <= A(0) and B(1);

T(2) <= A(0) and B(2);

T(3) <= A(0) and B(3);

T(4) <= A(1) and B(0);

T(5) <= A(1) and B(1);

T(6) <= A(1) and B(2);

T(7) <= A(1) and B(3);

T(8)  <= A(2) and B(0);

T(9)  <= A(2) and B(1);

T(10) <= A(2) and B(2);

T(11) <= A(2) and B(3);

A1 <= '0' & T(3) & T(2) & T(1);

B1 <= T(7) & T(6) & T(5) & T(4);

C1 <= T(11) & T(10) & T(9) & T(8);

O(0) <= T(0);

FA4_1 : FULL_ADDER_4BIT PORT MAP(A1,B1,'0',D1,C);

O(1) <= D1(0);

E1 <= C & D1(3 downto 1);

FA4_2 : FULL_ADDER_4BIT PORT MAP(E1,C1,'0',O(5 downto 2),O(6));

end Behavioral;

8. Design a single bit comparator cirucit using VHDL.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COMPARATOR_1BIT is

Port (	A, B : in std_logic;

X : out std_logic; -- A > B

Y : out std_logic; -- A < B

Z : out std_logic; -- A = B

EN : in std_logic);

end COMPARATOR_1BIT;

architecture Behavioral of COMPARATOR_1BIT is

begin

PROCESS(A,B,EN)

BEGIN

If (EN = '1') then

If (A > B) then

X <= '1'; Y <= '0'; Z <= '0';

elsif(A < B) then

X <= '0'; Y <= '1'; Z <= '0';

else

X <= '0'; Y <= '0'; Z <= '1';

end if;

else

X <= '0'; Y <= '0'; Z <= '0';

end if;

END PROCESS;

end Behavioral;

9. (a) Design a 4-bit comparator using the macro of single bit comparator.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COMPARATOR_4BIT_COMP is

Port ( 	P, Q : in std_logic_vector(3 downto 0);

M : out std_logic; -- P > Q

N : out std_logic; -- P < Q

O : out std_logic; -- P = Q

EN : in std_logic);

end COMPARATOR_4BIT_COMP;

architecture Behavioral of COMPARATOR_4BIT_COMP is

component COMPARATOR_1BIT is

Port (	A, B : in std_logic;

X : out std_logic; -- A > B

Y : out std_logic; -- A < B

Z : out std_logic; -- A = B

EN : in std_logic);

end component COMPARATOR_1BIT;

signal T : std_logic_vector(11 downto 0);

constant enable : std_logic := '1';

begin

c1: COMPARATOR_1BIT port map(P(3),Q(3),T(0),T(1),T(2),enable);

c2: COMPARATOR_1BIT port map(P(2),Q(2),T(3),T(4),T(5),T(2));

c3: COMPARATOR_1BIT port map(P(1),Q(1),T(6),T(7),T(8),T(5));

c4: COMPARATOR_1BIT port map(P(0),Q(0),T(9),T(10),T(11),T(8));

M <= T(0) OR T(3) OR T(6) OR T(9);

N <= T(1) OR T(4) OR T(7) OR T(10);

O <= T(11);

end Behavioral;

9. (b) Design a 4-bit comparator using the boolean function.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COMPARATOR_4BIT_BOOLEAN is

Port (	P, Q : in std_logic_vector(3 downto 0);

M : out std_logic; -- P > Q

N : out std_logic; -- P < Q

O : out std_logic; -- P = Q

EN : in std_logic);

end COMPARATOR_4BIT_BOOLEAN;

architecture Behavioral of COMPARATOR_4BIT_BOOLEAN is

begin

process(P,Q,EN)

begin

if(EN = '1')then

if(P > Q) then

M <= '1';

N <= '0';

O <= '0';

elsif(P < Q) then

M <= '0';

N <= '1';

O <= '0';

else

M <= '0';

N <= '0';

O <= '1';

end if;

else

M <= '0';

N <= '0';

O <= '0';

end if;

end process;

end Behavioral;

Set - III

======

1. Design a D-Flip flop with

a. Asynchronous preset and clear inputs.

b. Synchronous reset input.

c. Synchronous set input.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity D_F_F is

Port ( 	rst : in std_logic;

pr : in std_logic;

clr : in std_logic;

clk : in std_logic;

d : in std_logic;

q : out std_logic);

end D_F_F;

architecture Behavioral of D_F_F is

begin

process(rst,clk,pr,clr,d)

begin

if rst = '1' then

q <= '0';

elsif clr = '1'then

q <= '0';

elsif pr = '1'then

q <= '1';

elsif rising_edge(clk) then

q <= d;

end if;

end process;

end Behavioral;

2. Design a negative edge triggered JK flip-flop with asynchronous reset and preset inputs.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity JK_FF is

port(	J : in std_logic;

K : in std_logic;

PRESET: in std_logic;

CLEAR: in std_logic;

CLK : in std_logic;

Q: inout std_logic;

QBAR : out std_logic := '1');

end JK_FF;

architecture Behavioral of JK_FF is

begin

process(PRESET,CLEAR,CLK)

variable JK : std_logic_vector(1 downto 0):= J&K;

begin

if PRESET = '0' then

Q <= '1';

elsif CLEAR = '0' then

Q <= '0';

elsif CLK = '0' and CLK'event then

Q <= (J and not Q) or (not K and Q);

end if;

end process;

QBAR <= not Q;

end Behavioral;

- Write a code to madel a 4-bit shift register with serial input and parallel output data. The circuit has an active high asynchronous reset, data input Din, a shift enable input shift_en and 4-bit output shift_out.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity ShiftR_4bit_Sin_Pout is

Port(clk : in  std_logic;

rst : in  std_logic;

d : in    std_logic ;

q : out   std_logic_vector (3 downto 0);

en : in   std_logic);

end ShiftR_4bit_Sin_Pout;

architecture Behavioral of ShiftR_4bit_Sin_Pout is

signal q_s :  std_logic_vector(3 downto 0);

begin

process(clk,rst,d,en)

begin

if rst = '1' then

q_s <= (others => '0') ;

elsif rising_edge(clk) then

if en = '1' then

q_s <= d & q_s ( 3 downto 1) ;

end if;

end if;

end process;

q <= q_s;

end Behavioral;

4.Write a code to model 3-bit counter. The counter has an asynchronous input reset and synchronous input count_en. The counter start counting when coount_en = 1 on the rising edge of clock.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COUNTER3_BIT is

Port(rst : in std_logic;

clk : in std_logic;

preset : in std_logic;

count_enable : in std_logic;

aload_enable : in std_logic;

aload : in std_logic_vector(2 downto 0);

op : out std_logic_vector(2 downto 0));

end COUNTER3_BIT;

architecture Behavioral of COUNTER3_BIT is

signal c: std_logic_vector(2 downto 0);

begin

process(clk,rst,preset,aload_enable,count_enable)

begin

if rst = '0' then

c <= (others => '0');

elsif preset = '1' then

c <= (others => '1');

elsif  aload_enable = '1' then

c <= aload;

elsif rising_edge(clk) then

if count_enable = '1' then

c <= c + 1;

end if;

end if;

end process;

op <= c;

end Behavioral;

5.Develop a model of 4 bit up-down counter. It has asynchrounous reset input and synchronous inputs count_en and up. When synchronous input up is high, the counter counts up, when up is low it counts down.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COUNTER_UP_DOWN is

Port(	rst : in std_logic;

clk : in std_logic;

count_en : in std_logic;

up_dwn  : in std_logic;

op : out std_logic_vector(3 downto 0));

end COUNTER_UP_DOWN;

architecture Behavioral of COUNTER_UP_DOWN is

signal c: std_logic_vector(3 downto 0);

begin

process(rst,clk,up_dwn)

begin

if rst = '1'then

c <= (others => '0') ;

elsif rising_edge(clk)	then

if count_en = '1'then

if up_dwn = '1'then

c <= c + 1;

elsif up_dwn = '0'then

c <= c - 1;

end if;

end if;

end if;

end process;

op <= c;

end Behavioral;

6. Write a code to model 4 bit decade counter.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COUNTER_DACADE is

Port(	rst : in std_logic;

clk : in std_logic;

count_en : in std_logic;

op : out std_logic_vector(3 downto 0));

end COUNTER_DACADE;

architecture Behavioral of COUNTER_DACADE is

signal c: std_logic_vector(3 downto 0);

begin

process(rst,clk,count_en)

begin

if rst = '1'then

c <=  (others => '0') ;

elsif rising_edge(clk)	then

if count_en = '1'then

c <= c + 1;

end if;

if c  = "1001" then

c <= "0000";

end if;

end if;

end process;

op <= c;

end Behavioral;

7. Design a counter, which has to count up by one and down by two.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity COUNTER_UP1_DOWN2 is

Port(	rst : in std_logic;

clk : in std_logic;

count_en : in std_logic;

up_dwn  : in std_logic;

op : out std_logic_vector(3 downto 0));

end COUNTER_UP1_DOWN2;

architecture Behavioral of COUNTER_UP1_DOWN2 is

signal c: std_logic_vector(3 downto 0);

begin

process(rst,clk,up_dwn)

begin

if rst = '1'then

c <= (others => '0') ;

elsif rising_edge(clk)	then

if count_en = '1'then

if up_dwn = '1'then

c <= c + 1;

elsif up_dwn = '0'then

c <= c - 2;

end if;

end if;

end if;

end process;

op <= c;

end Behavioral;

8. Write a code to model a parallel to serial converter. The circuit accepts a 4 bit paralle data (data_in) when the synchronous load input goes high.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity ShiftR_Pin_Sout is

Port(clk : in  std_logic;

rst : in  std_logic;

d : in    std_logic_vector(3 downto 0) ;

q : out   std_logic;

en : in   std_logic);

end ShiftR_Pin_Sout;

architecture Behavioral of ShiftR_Pin_Sout is

signal st :  std_logic_vector(1 downto 0);

begin

process(clk,rst,d,en)

begin

if rst = '1' then

q <=  '0' ;

st <= "00";

elsif rising_edge(clk) then

if en = '1' then

case st is

when "00" =>

q <= d(3);

st <= "01";

when "01" =>

q <= d(2);

st <= "10";

when "10" =>

q <= d(1);

st <= "11";

when "11"	=>

q <= d(3);

st <= "00";

when others => null;

end case;

end if;

end if;

end process;

end Behavioral;